AgLogo Matlab Package (HOW-TO):

05.2018 Y. Murayama @ MPI-Tübingen

- Install

- Copy the entire folder of “//mpivision2/UsrLab/projects/MatLab_Package/MatLab” to your own place.

- See …/MatLab/readme.txt for installation.  Note that the package uses “startup.m” for setting up the path setting of MATLAB.

- Session/Description file

A so-called Session/Description file describes about information of one-day experiment (e.g. A13s11.m; where A13 as the animal ID, s11 as the unique ID).  A session can have several “groups” which contain scans under the same condition/stimuli.  GRP.(group-name).exps is an array of so-called experiment numbers which are arbitrary but uniquely associated to each scan as EXPP(expno).xxx.

- Data Organization

Data are stored in the session directory with sub-directories of “signals” from each acquisitions/scan of ephys recording or/and MRI.

For example of “A13s11”:

xxx/A13.s11/	cln/	a13s11_0001_cln.mat		% “Cln”   signal of ExpNo=1

a13s11_0002_cln.mat 	% “Cln”   signal of ExpNo=2

…

tcimg/	a13s11_0001_tcimg.mat	% “tcImg” signal of ExpNo=1

a13s11_0001_tcimg.mat	% “tcImg” signal of ExpNo=2

…

Note that most “signal” structures have fields of .dx (sampling time in sec) and .dat (data).

- Basic Neural signals are

Cln:  cleaned (denoised) ephys signals,

.dx : 	sampling time (sec)

.dat : 	data as [time, chan]

blp: band-limited power of “Cln”,

.dx : 	sampling time (sec)

.dat : 	data as [time, chan, bands]

.info:	band information (see . .info.blpinfo for each band)

- Baisc MRI signals are:

tcImg: volumes of fMRI images (min. processing with cropping/intensity normalization)

.dx : 	inter-volume time (sec)

.dat : 	data as [x, y, slice, time]

.ds :	voxel resolution as [xres, yres, zres] in mm

roiTs: processed fMRI data (ROI-wise time series with temporal/spatial filtering)

.dat :	data as [time, voxels],

.dx :	inter-volume time (sec),

.name:	ROI name

.coords :  XYZ coordinates in “tcImg” as [xyz, voxels]

- Data Loading

To load data (i.e. “signal”), use “sigload(Session,ExpNo,SignalName)” function.

>> Cln   = sigload('A13s11', 4, 'Cln');    % load “Cln”   of A13s11 (exp=4)

>> tcImg = sigload('A13s11', 5, 'tcImg');  % load “tcImg” of A13s11 (exp=5)